RB-DEED8 and RB-DEDD8

Command Protocol

©Sonifex Ltd, 2013 All Rights Reserved

Revision 1.00, November 2013

Sonifex Ltd, 61, Station Road, Irthlingborough,

Northants, NN9 5QE, England.

Tel: +44 (0)1933 650 700

Fax: +44 (0)1933 650 726

Email: sales@sonifex.co.uk

Website: http://www.sonifex.co.uk

Information in this document is subject to change without notice and does not represent a commitment on the part of the vendor. Sonifex Ltd shall not be liable for any loss or damage whatsoever arising from the use of information or any error contained in this document.

No part of this document may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying, recording, information storage and retrieval systems, for any purpose other than the purchaser’s personal use, without the express written permission of Sonifex Ltd. Unless otherwise noted, all names of companies, products and persons contained herein are part of a completely fictitious adaptation and are designed solely to document the use of Sonifex product.

Table of Contents

1 Notes	3

2 Common Commands	3

2.1 AMS	3

2.2 CMS	4

2.3 MAC	4

2.4 NET	5

2.5 NOP	5

2.6 PFR	6

2.7 RWC	6

2.8 SER	6

2.9 SRQ	6

2.10 UID	11

2.11 VER	11

2.12 WAL	11

2.13 WCG	12

2.14 WDT	12

2.15 WFL	12

2.16 WRA	13

2.17 WRE	13

2.18 WRP	13

3 RB-DEDD8 Commands	14

3.1 AMD	14

3.2 CMD	15

3.3 WAM	15

3.4 WCM	16

3.5 WCS	17

3.6 WMS	17

3.7 WPT	17

3.8 WRM	17

3.9 WSC	18

4 RB-DEDD8 Commands	18

4.1 DMS	18

4.2 PMR	19

4.3 WDC	19

4.4 WOS	21

4.5 WPM	21

5 Error Values	22

# Notes

TCP connections must be made to port 31780.

All commands are sent as strings.  These strings can be terminated with \r\n or \n but this is not necessary.

Unless otherwise stated, all numbers used in commands are in Hexadecimal format.

If a number (n) appears before a square bracket, then this data is repeated n times.  For example if a command appears as CMD:2[x,y,z,] then the actual command sent is CMD:x,y,z,x,y,z

# Common Commands

## AMS

Command:	AMS:p

Description:	Get Dolby E program AC-3 metadata status.  See Dolby documentation for 			definitions.

Parameters:	p = program number (0-7)

Returns:	AMS:p,t,26[a,]

p = program number (0-7)

t = Program description

a = AC-3 metadata values

The parameter values are returned in the following order :-

Dialogue normalization

Coding mode

LFE channel

Bit-stream mode

Line mode profile

RF mode profile

Center downmix level

Surround downmix level

Dolby surround mode

Audio production information exists

Mixing level

Room type

Extra bitstream information exists

Preferred downmix

Lt/Rt Center mix

Lt/Rt Surround mix

Lo/Ro Center mix

Lo/Ro Surround mix

Dolby Surround EX mode

DC Filter

Low-pass filter

LFE low-pass filter

Surround phase shift

Surround attenuation

## CMS

Command:	CMS:

Description:	Get common metadata status.

Parameters:	None

Returns:	[RB-DEED8]

CMS:a,b,c

a = Program configuration

0 = 5.1 + 2		1 = 5.1 + 2x1

2 = 4 + 4		3 = 4 + 2x2

4 = 4 + 2 + 2x1		5 = 4 + 4x1

6 = 4x2			7 = 3x2 + 2x1

8 = 2x2 + 4x1		9 = 2 + 6x1

10 = 8x1		11 = 5.1

12 = 4 + 2		13 = 4 + 2x1

14 = 3x2		15 = 2x2 + 2x1

16 = 2 + 4x1		17 = 6x1

18 = 4			19 = 2 + 2

20 = 2 + 2x1		21 = 4x1

22 = 7.1		23 = 7.1 Screen

b = Frame rate

1 = 23.98 fps

2 = 24 fps

3 = 35 fps

4 = 29.97 fps

5 = 30 fps

c = Bit depth

0 = 20-bit

1 = 16-bit

[RB-DEDD8]

CMS:a,b

a = Program configuration

See above.

b = Frame rate

See above.

## MAC

Command:	MAC:

Description:	Get unit MAC address.

Parameters:	None.

Returns:	Returns unit MAC address as a 12 character string

MAC:xxxxxxxxxxxx

## NET

Command:	NET:x,y,z	(SET)

NET:t		(GET)

Description:	Get static or current network address.

Set static address.  The addresses are sent/received in standard format i.e.				192.168.0.1

Example:	NET:192.168.1.28,255.255.255.0,192.168.1.1

Sets the static IP address to 192.168.1.28, the static subnet mask to 255.255.255.0 		and the static gateway to 192.168.1.1.

NET:0

Gets the static address settings.

Parameters:	GET operation

t = address type -

0 = static IP/gateway/subnet address

1 = current IP/gateway/subnet address

SET operation

x = IP address

y = Subnet mask

z = gateway

Returns:	Get operation

NET:t,x,y,z

Set operation

ACK: or

ERR:n

## NOP

Command:	NOP:x,y	(SET)

NOP:x		(GET)

Description:	Get/Set network options.  Currently only one option available.

Example:	NOP:0,1

Enables DHCP.

Parameters:	GET operation

x = option number

0 = DHCP enable/disable.

SET operation

x = option number

0 = DHCP enable/disable.

y = option value

0 = disable

1 = enable

Returns:	Get operation

NOP:x,y

Set operation

ACK: or

ERR:n

## PFR

Command:	PFR:

Description:	Performs factory reset.

Parameters:	None

Returns:	ACK:

## RWC

Command:	RWC:u,p

Description:	Request write permission.  This must be called once a connection has been made, to 		allow the connection to perform SET commands.  Not needed if only reading status.  		Write permission can only be granted to a single connection at any one time.

Parameters:	u = user name

p = password

Returns:	ACK: on success

ERR:n or

NAK: when user name/password is incorrect or write permission has already been 		granted to another connection.

## SER

Command:	SER:

Description:	Get unit serial number.

Parameters:	None

Returns:	Returns unit serial number.

SER:xxxxxx

## SRQ

Command:	SRQ:t

Description:	Get unit status.

Parameters:	t = status group (0-5)

[RB-DEED8]

0 = System status

2 = Encoder

3 = External audio I/O

4 = Encoder inputs

5 = Remotes

[RB-DEDD8]

0 = System status

2 = Decoder

3 = External audio I/O

4 = Decoder outputs

5 = Remotes

Returns:	[RB-DEED8]

System Status

SRQ:0,a,b,c,d,e,f

a = Metadata

b = Encode

c = Control lock

d = Video ref lock

e = Video ref rate

f = Channel activity

Encoder

SRQ:2,a,b,c,d,e,f

a = Encoding mode

0 = Dolby E encoding

1 = Pass-through

3 = Dolby E encoding test -18dBFS

4 = Pass-through test -18dBFS

5 = Dolby E encoding test -20dBFS

6 = Pass-through test -20dBFS

7 = Dolby E encoding test silence

8 = Pass-through test silence

b = Pass-through latency

0 = Normal

1 = Minimum

c = Pass-through SRC

0 = Enabled

1 = Disabled

d = Output clock source

0 = Video frame sync

1 = Internal clock

2 = Input CH1/2

e = Metadata source

0 = External

1 = Internal

f = Reversion mode

0 = Last used

1 = Internal

2 = Stop encoding

External audio I/O

SRQ:3,a

a = External audio termination

bit 0 = channel 1/2 (0 = 75 Ohm, 1 = 110 Ohm)

bit 1 = channel 3/4 (0 = 75 Ohm, 1 = 110 Ohm)

bit 2 = channel 5/6 (0 = 75 Ohm, 1 = 110 Ohm)

bit 3 = channel 7/8 (0 = 75 Ohm, 1 = 110 Ohm)

bit 4 = Dolby output 1 (0 = 75 Ohm, 1 = 110 Ohm)

bit 5 = Dolby output 2 (0 = 75 Ohm, 1 = 110 Ohm)

Encoder inputs

SRQ:4,8[a],b

a = Channel gain for each available channel.

0 = -24 dB

1 = -23.5 dB...

48 = 0 dB

49 = 0.5 dB...

96 = +24 dB

b = reserved

Remotes

SRQ:5,8[a]

a = Remote output setting. See WRA, WRE and WRP for definitions.

bit 0 = polarity

bit 1 = action

bits 7-3 = event

[RB-DEDD8]

System status

SRQ:0,a,b,c,d,e,f,g

a = Bit-stream format

0 = Dolby Digital 32-bit

1 = Dolby Digital 16-bit (channel 1)

2 = Dolby Digital 16-bit (channel 2)

3 = Dolby Digital 16-bit (channels 1 and 2)

4 = Dolby E 24-bit

5 = Dolby E 20-bit

6 = Dolby E 16-bit

7 = PCM

b = video sync status

0 = OK

1 = error

c = Frame rate

1 = 23.98 fps

2 = 24 fps

3 = 25 fps

4 = 29.97 fps

5 = 30 fps

d = Front panel lock status

0 = Not locked

1 = locked

e = Video ref lock

f = Video ref rate

g = Output activity

Decoder

SRQ:2,18[a,]

a = Decoder parameters listed in the following order.

Reserved

Reserved

Reserved

AES channel select

0 = AES channel 1

1 = AES channel 2

Stream select

0 = Auto

1 = Stream select 1

2 = Stream select 2

3 = Stream select 3

4 = Stream select 4

5 = Stream select 5

6 = Stream select 6

7 = Stream select 7

Listening mode

0 = Full

1 = Dolby Digital Surround EX

2 = 3 Stereo

3 = Phantom

4 = Stereo

5 = Mono

DRC mode

0 = Line

1 = RF

2 = Custom

3 = Bypass

DRC custom low level boost (0-10)

DRC custom high level cut (0-10)

DRC line low level boost (0-10)

DRC line high level cut (0-10)

Dialogue normalization

0 = disabled

1 = enabled

Pro-logic decode

0 = disabled

1 = enabled

PCM latency

0 = single frame

1 = minimum

PCM metadata enable/disable

0 = disabled

1 = enabled

AUX output mode

0 = Lt/Rt

1 = Lo/Ro

2 = Mono

3 = Mute

AUX DRC

0 = Line

1 = RF

AUX Dolby E program select (0-7)

External audio I/O

SRQ:3,10[a],b

a = Output channel routing for each output channel

0 = Decoder output channel 1

1 = Decoder output channel 2

2 = Decoder output channel 3

3 = Decoder output channel 4

4 = Decoder output channel 5

5 = Decoder output channel 6

6 = Decoder output channel 7

7 = Decoder output channel 8

8 = Decoder AUX left

9 = Decoder AUX right

10-31 = Silence

b = External audio termination

bit 0 = Dolby input (0 = 75 Ohm, 1 = 110 Ohm)

bit 1 = channel 1/2 (0 = 75 Ohm, 1 = 110 Ohm)

bit 2 = channel 3/4 (0 = 75 Ohm, 1 = 110 Ohm)

bit 3 = channel 5/6 (0 = 75 Ohm, 1 = 110 Ohm)

bit 4 = channel 7/8 (0 = 75 Ohm, 1 = 110 Ohm)

bit 5 = Dolby output (0 = 75 Ohm, 1 = 110 Ohm)

bit 6 = AUX 1/2 (0 = 75 Ohm, 1 = 110 Ohm)

Decoder outputs

SRQ:4,10[a]

a = Channel gain

0 = -24 dB

1 = -23.5 dB...

48 = 0 dB

49 = 0.5 dB...

96 = +24 dB

Remotes

SRQ:5,8[a]

a = Remote output setting. See WRA, WRE and WRP for definitions.

bit 0 = polarity

bit 1 = action

bits 7-3 = event

## UID

Command:	UID:

Description:	Get unit ID.

Parameters:	None

Returns:	[RB-DEED8]

UID:RB-DEED8

[RB-DEDD8]

UID:RB-DEDD8

## VER

Command:	VER:

Description:	Get version information.  Note that version numbers are in decimal format.

Parameters:	None

Returns:	VER:x.x.x,y.y.y,z.z.z.z

x = Firmware version

y = FPGA version

z = Dolby OEM version

## WAL

Command:	WAL:u,p

Description:	Set user name and password for write permission.  Default user name is “admin” 		and default password is “admin”.

Parameters:	u = user name

p = password

Returns:	ACK: or

ERR:n

## WCG

Command:	WCG:x,y

Description:	Set channel gain in 0.5 dB increments.

Parameters:	x = channel (0-25)

y = gain value (0-96)

0 = -24 dB

1 = -23.5 dB...

48 = 0 dB

49 = 0.5 dB...

96 = +24 dB

Returns:	ACK: or

ERR:n

## WDT

Command:	WDT:c,o

Description:	Set digital I/O termination.

Parameters:	c = external audio connection number.

[RB-DEDD8] (0-5)

0 = Dolby input

1 = Channel 1/2

2 = Channel 3/4

3 = Channel 5/6

4 = Channel 7/8

5 = Dolby output

6 = AUX output

[RB-DEED8] (0-4)

0 = Channel 1/2

1 = Channel 3/4

2 = Channel 5/6

3 = Channel 7/8

4 = Dolby output 1

5 = Dolby output 2

o = option (0-1)

0 = 75 Ohm

1 = 110 Ohm

Returns:	ACK: or

ERR:n

## WFL

Command:	WFL:x

Description:	Get/Set front panel lock.

Parameters:	x = enable/disable

0 = disable

1 = enable

Returns:	ACK: or

ERR:n

## WRA

Command:	WRA:o,i,e

Description:	Set remote I/O action.

Parameters:	o = remote number (0-7)

i = input or output (0-1)

0 = input

1 = output

e = action (0-1)

0 = momentary

1 = latched

Returns:	ACK: or

ERR:n

## WRE

Command:	WRE:o,i,e

Description:	Set remote I/O event.

Parameters:	o = remote number (0-7)

i = input or output (0-1)

0 = input

1 = output

e = event

[RB-DEDD8] (0-6)

0 = none

1 = decoder error

2 = decoding Dolby E

3 = decoding Dolby D

4 = decoding PCM

[RB-DEED8] (0-4)

0 = none

1 = encoder error

2 = external/VANC metadata

3 = encoding stopped

Returns:	ACK: or

ERR:n

## WRP

Command:	WRP:o,i,e

Description:	Set remote I/O polarity.

Parameters:	o = remote number (0-7)

i = input or output (0-1)

0 = input

1 = output

e = polarity (0-1)

0 = normally active

1 = normally inactive

Returns:	ACK: or

ERR:n

# RB-DEDD8 Commands

## AMD

Command:	AMD:p

Description:	Get Program AC3 Metadata (Internal)

Parameters:	p = Program number (0-7)

Returns:	AMD:p,t,24[a]

p = Program number (0-7)

t = Program description

a = AC-3 metadata settings

Dialogue normalization

Coding mode

LFE channel

Bit-stream mode

Line mode profile

RF mode profile

Center downmix level

Surround downmix level

Dolby surround mode

Audio production information exists

Mixing level

Room type

Extra bitstream information exists

Preferred downmix

Lt/Rt Center mix

Lt/Rt Surround mix

Lo/Ro Center mix

Lo/Ro Surround mix

Dolby Surround EX mode

DC Filter

Low-pass filter

LFE low-pass filter

Surround phase shift

Surround attenuation

## CMD

Command:	CMD:

Description:	Get Common Metadata (Internal)

Parameters:	None

Returns:	CMD:p,f,b

p = program configuration

0 = 5.1 + 2		1 = 5.1 + 2x1

2 = 4 + 4		3 = 4 + 2x2

4 = 4 + 2 + 2x1		5 = 4 + 4x1

6 = 4x2			7 = 3x2 + 2x1

8 = 2x2 + 4x1		9 = 2 + 6x1

10 = 8x1		11 = 5.1

12 = 4 + 2		13 = 4 + 2x1

14 = 3x2		15 = 2x2 + 2x1

16 = 2 + 4x1		17 = 6x1

18 = 4			19 = 2 + 2

20 = 2 + 2x1		21 = 4x1

22 = 7.1		23 = 7.1 Screen

f = frame rate

1 = 23.98 fps

2 = 24 fps

3 = 35 fps

4 = 29.97 fps

5 = 30 fps

b = bit depth

0 = 20-bit

1 = 16-bit

## WAM

Command:	WAM:p,a,b

Description:	Set Program AC3 Metadata (Internal)

Parameters:	p = program number (0-7)

a = parameter number (0-23)

0 = Dialogue normalization

1 = Coding mode

2 = LFE channel

3 = Bitstream mode

4 = Line mode profile

5 = RF mode profile

6 = Center downmix level

7 = Surround downmix level

8 = Dolby surround mode

9 = Audio production information exists

10 = Mixing level

11 = Room type

12 = Extra bitstream information exists

13 = Preferred downmix

14 = Lt/Rt Center mix

15 = Lt/Rt Surround mix

16 = Lo/Ro Center mix

17 = Lo/Ro Surround mix

18 = Dolby Surround EX mode

19 = DC Filter

20 = Lowpass filter

21 = LFE lowpass filter

22 = Surround phase shift

23 = Surround attenuation

b = parameter value

See Dolby documentation.

Returns:	ACK: or

ERR:n

## WCM

Command:	WCM:p,d

Description:	Set Common Metadata (Internal)

Parameters:	p = parameter number (0-2)

0 = program configuration

1 = frame rate

2 = bit-depth

d = parameter value

Program Configuration

0 = 5.1 + 2		1 = 5.1 + 2x1

2 = 4 + 4		3 = 4 + 2x2

4 = 4 + 2 + 2x1		5 = 4 + 4x1

6 = 4x2			7 = 3x2 + 2x1

8 = 2x2 + 4x1		9 = 2 + 6x1

10 = 8x1		11 = 5.1

12 = 4 + 2		13 = 4 + 2x1

14 = 3x2		15 = 2x2 + 2x1

16 = 2 + 4x1		17 = 6x1

18 = 4			19 = 2 + 2

20 = 2 + 2x1		21 = 4x1

22 = 7.1		23 = 7.1 Screen

Frame rate

1 = 23.98 fps

2 = 24 fps

3 = 35 fps

4 = 29.97 fps

5 = 30 fps

Bit-depth

0 = 20-bit

1 = 16-bit

Returns:	ACK: or

ERR:n

## WCS

Command:	WCS:c,s

Description:	Set output clock source.

Parameters:	s = clock source (0-2)

0 = Video ref

1 = Internal

2 = Input CH1/2

Returns:	ACK: or

ERR:n

## WMS

Command:	WMS:s

Description:	Set Metadata Source

Parameters:	s = metadata source (0-10)

0 = External

1 = Internal

## WPT

Command:	WPT:p,t

Description:	Set Program Description Text (Internal)

Parameters:	p = program number (0-7)

t = program text description (32 characters max)

Returns:	ACK: or

ERR:n

## WRM

Command:	WRM:m

Description:	Set Reversion Mode

Parameters:	m = reversion mode (0-2)

0 = Last used

1 = Internal

2 = Stop encoding

Returns:	ACK: or

ERR:n

## WSC

Command:	WSC:p,d

Description:	Set Encoder Sys Control

Parameters:	p = parameter number (0-2)

0 = Encoding mode

1 = Pass-through latency

2 = Pass-through SRC

d = parameter value

if p = 0

0 = Dolby E encoding

1 = Pass-through

3 = Dolby E encoding test -18dBFS

4 = Pass-through test -18dBFS

5 = Dolby E encoding test -20dBFS

6 = Pass-through test -20dBFS

7 = Dolby E encoding test silence

8 = Pass-through test silence

else if p = 1

0 = Normal

1 = Minimum

else if p = 2

0 = Enabled

1 = Disabled

Returns:	ACK: or

ERR:n

# RB-DEDD8 Commands

## DMS

Command:	DMS:

Description:	Get Dolby Digital AC3 Metadata Status.  See Dolby documentation for parameter 		values and definitions.

Parameters:	None

Returns:	DMS:22[a,]

a = metadata status listed in the following order

Data rate

Dialogue normalization

Coding mode

LFE channel

Bit-stream mode

Dynamic range

Compression

Center mix level

Surround mix level

Dolby Surround mode

Audio production information exists

Mix level

Room type

Extra bit stream information exists

Preferred downmix

Lt/Rt Center mix

Lt/Rt Surround mix

Lo/Ro Center mix

Lo/Ro Surround mix

Dolby Surround EX mode

## PMR

Command:	PMR:N[p]

Description:	Get PCM Metadata

Parameters:	p = parameter

2 = bit-stream mode

3 = coding mode

6 = Dolby surround mode

11 = audio production information exists

12 = mixing level

13 = room type

28 = DC filter

29 = low-pass filter

35 = RF mode profile

38 = line mode profile

Returns:	PMR:N[p,d]

p = parameter

See the parameter section above.

d = parameter value

See Dolby documentation for parameter values and definitions.

## WDC

Command:	WDC:p,d

Description:	Set Decoder Control

Parameters:	p = parameter number, d = parameter value

3 = AES channel select

0 = AES channel 1

1 = AES channel 2

4 = stream select

0 = Auto

1 = Stream select 1

2 = Stream select 2

3 = Stream select 3

4 = Stream select 4

5 = Stream select 5

6 = Stream select 6

7 = Stream select 7

5 = listening mode

0 = Full

1 = Dolby Digital Surround EX

2 = 3 Stereo

3 = Phantom

4 = Stereo

5 = Mono

6 = DRC mode

0 = Line

1 = RF

2 = Custom

3 = Bypass

7 = DRC custom low level boost (0-10)

8 = DRC custom high level cut (0-10)

9 = DRC line low level boost (0-10)

10 = DRC line high level cut (0-10)

12 = Dialogue normalization

0 = disabled

1 = enabled

13 = Pro-logic decode

0 = disabled

1 = enabled

14 = PCM latency

0 = single frame

1 = minimum

15 = PCM metadata enable/disable

0 = disabled

1 = enabled

16 = AUX output mode

0 = Lt/Rt

1 = Lo/Ro

2 = Mono

3 = Mute

17 = AUX DRC

0 = Line

1 = RF

18 = AUX Dolby E program select (0-7)

Returns:	ACK: or

ERR:n

## WOS

Command:	WOS:c,s

Description:	Routes a channel to an external audio output.

Parameters:	c = external audio output channel (0-9)

0 = channel 1

1 = channel 2

2 = channel 3

3 = channel 4

4 = channel 5

5 = channel 6

6 = channel 7

7 = channel 8

8 = AUX left

9 = AUX right

s = source (0-31)

0 = Decoder output channel 1

1 = Decoder output channel 2

2 = Decoder output channel 3

3 = Decoder output channel 4

4 = Decoder output channel 5

5 = Decoder output channel 6

6 = Decoder output channel 7

7 = Decoder output channel 8

8 = Decoder AUX left

9 = Decoder AUX right

10-31 = Silence

Returns:	ACK: or

ERR:n

## WPM

Command:	WPM:p,d

Description:	Set PCM Metadata

Parameters:	p = parameter number

2 = bit-stream mode

3 = coding mode

6 = Dolby surround mode

11 = audio production information exists

12 = mixing level

13 = room type

28 = DC filter

29 = low-pass filter

35 = RF mode profile

38 = line mode profile

d = parameter value

See Dolby documentation for parameter values and definitions.

Returns:	ACK: or

ERR:n

# Error Values

When an error occurs on a particular command.  ERR: will be returned along with an error code.  These codes are described below.

0 = no error.

2 = unknown command.

3 = incorrect number of parameters.

4 = one or more of the parameters contained an invalid value.

6 = read only.  A SET command was sent when the connection has no write permission.